Arduino BLE controlled obstacle avoiding car

Abstract:

This project describes about an obstacle avoiding car who’s basic idea is to follow a fixed path along with avoiding any obstacles which it encounters on its way by setting a threshold distance  between the car and the obstacle which will be controlled by IR , ULTRASONIC and LDR sensors , we will use Arduino ATMega328P and the programming language used to achieve this will be Embedded C programming and the software used is Arduino IDE .We will be using wireless signal communication with 2 Bluetooth Modules which work on the principle of Master and Slave communication ,where  Master is mode 1 and Slave is mode 0,one  Bluetooth Module will be on the car which will act as a receiver and the other will act as a Transmitter and will be with us , we will also add an LED on the car which will resemble the headlight of a car and a buzzer for the horn functionality

Literature Review:

| Patent Application Number | Patent Title | Country | Description |
| --- | --- | --- | --- |
| 202421087952 | Obstacle Circumvention Car Using Arduino | India | The Obstacle Circumvention Car is an autonomous robotic vehicle that detects and avoids obstacles using ultrasonic sensors and a microcontroller. It processes real-time sensor data and adjusts its path using pre-programmed algorithms. |
| Paper details |  |  |  |
| Inventor and Applicant: GOPAL DADARAO UPADHYE ADVAIT MHALUNGEKAR GAURAV DESALE PURVA RATNAPARKHI GAYATRI BHURGUDA, VISHWAKARMA INSTITUTE OF TECHNOLOGY ,666, UPPER INDIRA NAGAR, BIBWEWADI, PUNE, MAHARASHTRA, INDIA- 411037 They worked on Autonomous car that detects obstacles, navigates safely using sensors and algorithms. |  |  | A motor driver unit controls movement, ensuring smooth navigation. This cost-effective and modular system is ideal for robotics and IoT applications. Its ability to operate safely in various environments makes it suitable for automation and smart transport solutions. |

| Patent Application Number | Patent Title | Country | Description |
| --- | --- | --- | --- |
| 202441101716 | Smart Obstacle-Avoiding Vehicle | India | The Obstacle Avoiding Robot Car is an autonomous vehicle designed to detect and navigate around obstacles using ultrasonic sensors. It employs an Arduino microcontroller and three ultrasonic distance sensors to enhance accuracy in obstacle detection. |
| Paper details |  |  |  |
| Inventor and Applicant: Dr. R. Umamaheshwari Mrs. A. Sravan V. Lakitha U. Sai Gayatri Vashitha P. Shyama Yashitha N. Jayavardhan Vabbina Syam Prasad Vignan’s Institute of Information Technology, Besides VSEZ Vadlapudi Duvvada, Gajuwaka, Visakhapatnam, Andhra Pradesh-530049 They worked on Autonomous robot car using Arduino, ultrasonic sensors for obstacle avoidance. |  |  | The robot processes real-time data and adjusts its movement accordingly, ensuring smooth navigation without collisions. Arduino software is used for programming, making it efficient and easy to develop. With widely available and inexpensive hardware, this project offers a cost-effective and easily replicable solution for autonomous robotics applications. |

| Patent Application Number | Patent Title | Country | Description |
| --- | --- | --- | --- |
| 202441084569 | Autonomous Obstacle-Avoiding Vehicle Using Arduino Technology | India | The Obstacle-Avoiding Vehicle uses Arduino technology and ultrasonic sensors to detect and navigate around obstacles autonomously. The Arduino microcontroller processes real-time sensor data and controls a motor driver circuit to adjust movement, ensuring collision-free navigation. |
| Paper details |  |  |  |
| Inventor and Applicant: Devineni Gireesh Kumar Md. Khaja Farman Jarabana Nikhila K Sahithi N Aashish V Shanthan G Ramani E Thrisha Ch. Shiva Charan Reddy B V RAJU INSTITUTE OF TECHNOLOGY Vishnupur, Narsapur, Medak, Telangana 502313 They worked on Arduino vehicle using sensors to autonomously avoid obstacles and navigate. |  |  | This cost-effective system demonstrates the potential of robotics in automation and smart transportation. Future enhancements may include LIDAR or cameras for improved navigation. |

| Patent Application Number | Patent Title | Country | Description |
| --- | --- | --- | --- |
| 202411067715 | OBSTACLE DETECTION AND AVOIDANCE SYSTEM | India | The invention presents an advanced obstacle detection and avoidance system for self-driving vehicles using an Arduino Uno R3 microcontroller. A triangular sensor arrangement with three ultrasonic sensors enhances detection accuracy and response time. |
| Paper details |  |  |  |
| Rushil Gupta Garima Gupta Rushil Gupta & Garima Gupta, Address of the Applicant: H-65, Sector XI, Noida, Uttar Pradesh, 201301. They worked on Arduino-powered vehicle using ultrasonic sensors for autonomous obstacle avoidance navigation. |  |  | The system, powered by a 12V battery and controlled via an L298N motor driver, enables smooth navigation by stopping, reversing, and turning based on real-time data. This cost-effective solution ensures safe and reliable autonomous movemet.. |

| Patent Application Number | Patent Title | Country | Description |
| --- | --- | --- | --- |
| 202441057952 | OBSTACLE AVOIDING ROBOT USING ARDUINO | India | Robotics is a rapidly growing and innovative field aimed at simplifying human life. Autonomous robots can navigate unknown environments independently using software intelligence and sensor-based obstacle detection. This project focuses on designing a robotic car equipped with infrared sensors to detect and avoid obstacles in real-time, ensuring smooth movement. By integrating this technology into transportation systems, vehicles can automatically detect obstacles and adjust their path, reducing the risk of accidents. This advancement has potential applications in smart transportation, automation, and safety systems. |
| Paper details |  |  |  |
| T. Selvaganapathi S.M.Sairam G.Ajay S.Divagar Karpagam Academy of Higher Education, Karpagam Institute of Technology Pollachi Main Road, Eachanari Post, Coimbatore-641 021, Tamil Nadu, India They worked on Autonomous robotic car using infrared sensors for real-time obstacle avoidance. |  |  |  |

Flow Chart:

Block Diagram:

Components list:

Component 1- Arduino ATMega328P

An Arduino board is an open-source microcontroller platform used for building electronics projects. It allows users to easily program and control devices like sensors, motors, and LEDs through its simple interface and wide community support. Arduino boards can be connected to a computer via USB for programming and can operate autonomously once programmed.

Component 2- Ultrasonic Sensor

An ultrasonic sensor is a kind of electronic device that uses ultrasonic sound waves to detect the distance between two objects and transforms the reflected sound into an electrical signal. Ultrasonic waves move quicker than audible sound (i.e., the sound that humans can hear).

Component 3- Breadboard

A breadboard, solderless breadboard, or protoboard is a construction base used to build semi-permanent prototypes of electronic circuits. Unlike a perfboard or stripboard, breadboards do not require soldering or destruction of tracks and are hence reusable.

Component 4- Jumper Wires

Jumper wires are simply wires that have connector pins at each end, allowing them to be used to connect two points to each other without soldering. Jumper wires are typically used with breadboards and other prototyping tools in order to make it easy to change a circuit as needed.

Component 5- Chassis wheels motors

The wheel suspension is part of the chassis. The chassis comprises the following components: wheels, wheel carrier, wheel bearing, brake, wheel suspension, axle support, suspension (including anti-roll bar)... Coupling rod. Hydraulic shock absorber.

Component 6- LDR sensor

A light-dependent resistor (LDR) sensor is a passive electronic device that detects light and changes its resistance based on the amount of light. LDRs are also known as photo resistors.

Component 7- LEDs, resistors

The resistors and LEDs are the most essential electronic components which forms the basis of most of the electrical or electronics circuit. The main function of an LED or Light Emitting Diode is to emit light once electric current is supplied through it whereas a resistor is used to restrict the flow of current.

Component 8- TFT SPFD5408 LCD

The TFT SPFD5408 LCD is a 2.4-inch color display with a resolution of 240x320 pixels. It's often used in Arduino projects and typically includes a 4-wire resistive touchscreen.

Working operation:

The working operation describes how the obstacle-avoiding car functions. Here's a detailed breakdown:

Obstacle Detection:

1)The car uses sensors to scan its environment for obstacles.

2)Ultrasonic sensors emit sound waves and measure the time it takes for the waves to return, calculating the distance to objects.

3)A threshold distance is set, and when an obstacle is detected within this threshold, the car initiates its avoidance maneuver.

Data Processing:

1)The Arduino ATMega328P microcontroller acts as the "brain" of the car.

2)It receives the distance data from the sensors.

3)The Arduino is programmed using Embedded C in the Arduino IDE to process this data.

4)The program contains the logic to determine the car’s response (e.g., stop, turn, reverse) based on the sensor readings.

Movement Control:

1)The Arduino sends signals to the motor driver circuit.

2)The motor driver controls the car's motors, which in turn control the wheels.

3)This enables the car to perform the necessary actions to avoid the obstacle (e.g., turning away from the obstacle).

4)User Interaction and Control

The car can be operated in two modes

Bluetooth Control:

1)Bluetooth modules are used for wireless communication.

2)One Bluetooth module acts as a transmitter (controlled by the user), and the other acts as a receiver (on the car).

3)This allows the user to remotely control the car's movement.

LCD Control:

1)A TFT LCD is integrated into the car.

This enables direct, local control and display of information on the car itself.

2)Additional Features:

3)An LED is included to function as a headlight.

4)A buzzer is used for horn functionality.

5)A power supply module provides DC power to the car's electronic components.

Features (Advantages compare to other works):

1)Obstacle Avoidance: The core feature is the ability to autonomously avoid obstacles.

2)Microcontroller-Based Control: Utilizes an Arduino ATMega328P for processing and control.

3)Real-time Data Processing: Sensor data is processed in real-time for navigation decisions.

4)Sensor Fusion: Employs a combination of different sensor types (Ultrasonic) to enhance obstacle detection.

5)Wireless Control: Incorporates Bluetooth modules for remote, wireless control of the car.

6)Local Control and Display: Integrates a TFT LCD for direct control and information display on the car.

7)Potential: The technology has potential in smart transport.

8)Dual Operation Modes: Offers the flexibility to operate the car using either Bluetooth or the LCD.

9)Visual Indication: Includes an LED to simulate a headlight.

10)Audible Indication: Includes a buzzer for horn functionality.

11)Modular Design: The use of components like breadboards and jumper wires facilitates easy prototyping and modification.

Confines (Drawbacks or limitations):

As before:

Sensor Limitations: Ultrasonic sensors may have limitations in certain environments.

Need for Enhanced Navigation: There's a recognized need for potentially more advanced sensors like LIDAR or cameras for better navigation.

Requirement of customized Microcontroller board: Since there are 4 motors of 5volts each which requires a lot of power but the Arduino can only be given a maximum of 12Volts for optimum operation.

Adding a limitation based on the dual-mode operation:

Complexity: Integrating both TFT LCD and Bluetooth control adds complexity to the system's design and software, which might increase development time and potential points of failure.

Partial demo:

Source Code

1)Bluetooth:

2) TFT SPFD5408 LCD:

THANK YOU